# Data preparation: summary and stepwise instructions

All properties in Scotland must obtain an Energy Performance Certificate (EPC) when built, sold, or rented. This includes an assessment of the property’s energy efficiency along with other details such as property type, floor size, and potential retrofit measurers. The Scottish Government publish extracts from the Scottish EPC Register quarterly, dating back to 2008 for domestic properties and 2013 for non-domestic properties.

The EPC Register is the only known open access source of property level energy data for Scotland. Further, the property identifier included in the dataset allows it to be combined with the open access map of Scotland’s Unique Property Reference Numbers (UPRNs) to allow for spatial analysis. While both the UPRN map and EPC Register have known limitations, this level of detail is valuable in helping us to understand the quality, type, and typical energy consumption of buildings present in an area. An essential for energy planning.

Preparation of this data for use in CATHeaPS_FATCAT requires seven processing steps, as follows:

- Obtain UPRN data and trim for the study area

- Identify building-related UPRN points

- Join data zone and post code data to the UPRN points

- Obtain EPC data and populate the UPRN map with available EPC data

- Estimating energy and retrofit data for properties not included in the EPC Register

- Validate demand data

- Combine property level data into building level data

Steps 1 to 3 are carried out using a geoprocessing package such as ArcGIS or QGIS. Steps 4 to 7 are carried out in the Excel-based data processing model. A full breakdown of each step is set out below.

1. Obtain UPRN map for area

- Download latest open access UPRN file and save in folder. The files are available at: https://osdatahub.os.uk/downloads/open/OpenUPRN. Downloading in GeoPackage format is recommend for ease of use.

- Start a new project in ArcGIS Pro (or QGIS) and import the GeoPackage using the Add Data function in the Map ribbon.

- Search for the Glasgow_City_Council_Boundary in the ArcGIS Online Portal which can be found in the Catalog window and add this to your map.

- Use the Intersect tool to create a new layer with the UPRN points for Glasgow City Council only.

- Use the lasso select tool to select the area to model and save the selected data as a new layer file. For this study we select all points within the Glasgow City Council boundary that lie to the north of the canal that runs south of Drumchapel and name the new layer file “uprn_gcc drumchapel”.

2. Identification of building-related UPRN points

- Search for the OSOpenMapLocalBuildings layer package in the ArcGIS Online Portal and add it to the map. This includes a Local Buildings layer and a Local Important Buildings layer.

- Use the Spatial Join tool to join the Local Buildings layer to the Target Features within 0 meters “uprn_gcc drumchapel”, naming the output feature class “uprn_gccdrumchap_buildings”.

- Repeat this step for the Local Important Buildings layer using the Target Features within 0 meters “uprn_gccdrumchap_buildings”, naming the output feature class “uprn_gccdrumchap_buildingsimportant”.

3. Join datazone and postcode data

- Search for DataZoneBdry2011 in the ArcGIS Online Portal and add the layer to the map.

- Use the Spatial Join tool to join the layer to the Target Features “uprn_gccdrumchap_buildingsimportant”, naming the output feature class “uprn_gccdrumchap_dz”.

- Search for Postcodes_Scotland in the ArcGIS Online Portal and add the layer to the map.

- Use the Spatial Join tool to join the layer to the Target Features “uprn_gccdrumchap_dz”, naming the output feature class “uprn_gccdrumchap_pc”.

- Use the Table to Excel tool to save the “uprn_gccdrumchap_pc” table as an excel file.

4. Obtain EPC data and populate the UPRN map with available EPC data

- Download latest EPC files and save in folder. The files are available at: https://www.scottishepcregister.org.uk/CustomerFacingPortal/DataExtract

- Use the Property Data Processing excel file to compile the relevant EPC data.

- Before running the macro, paste the UPRN list from Step 3e in the “uprns” tab.

- Run the macros to populate the EPC_domestic and EPC_nondomestic tabs in the workbook. These look through the EPC files, store the rows that have a UPRN on our list, filter out any duplicates to use the latest EPC entry, and then paste the data into the relevant tab.

5. Estimating energy and retrofit data for properties not included in the EPC register

- The missing energy data is estimated in the workbook as follows:

- Property type:

  - Count number of properties at each grid reference

  - If count = 1: property is house

  - If count > 1: allocate each UPRN at grid reference a position number

    - If UPRN is in final position: assume point related to close services (i.e. not a property) and remove

    - Else: property is a flat

- All other data fields are assumed to equal that of the most recent EPC of a close-by property of the same property type. Close by properties are found by searching the data in the same grid reference, post code area, and data zone, with the smallest area is used where known data exists.

6. Validation of demand data

- The total heating demand for each datazone is compared with data obtained from the ScotHeatMap, available at: https://heatmap.data.gov.scot/custom/heatmap/.

- Large differences are investigated and the Manual_Overrides tab used to apply corrections at property level.

7. Combine the data at property level to generate a new dataset at building level

- Properties with the same grid reference are assumed to be part of the same building. The data fields are summed by grid reference to generate the building level data. Where property types at one grid reference are a mixture of residential and non-residential, this building is assumed to be a residential one.

- Data from the OUTPUT All buildings data can now be pasted directly into CATHeaPS-FatCat.